2011 CTD Data Files

IOS Cruise Number 2011-20

Cruise

There were 52 CTD casts performed aboard the CCGS Louis S. St-Laurent  between 21 July and 18 August, 2012 in the Canada Basin using a SBE911+ CTD system with 24 10-L Niskins.   XCTDs, vertical net tows for zooplankton, moorings and ice-tethered profilers were also conducted, deployed and/or recovered during this trip.

CTD Data Summary

The CTD data for down and upcasts are provided in 1-db averaged files (*.cnv),  in Seabird’s text format with one file per cast and separate files for down and up direction.  The files contain both processed and unprocessed variables, described below.  The downcast files are the primary data set.  The upcast files are provided for reference only.  They are useful for confirming unusual features seen in the downcast.

Downcast

Filename:  d201120_xxxx.cnv where xxxx is cast number

Standard seabird processing steps were used.

Pressure, primary and secondary temperature, primary and secondary conductivity, oxygen and fluorescence have been calibrated.  Transmissivity drift was corrected.

Spikes in primary temperature and primary conductivity have been interpolated over and where needed secondary values (when available) have replaced the primary values.

Derived variables, salinity, potential temperature, sigma-theta and sound velocity, 	were recalculated.

Transmission, CDOM fluorescence, altimetry, PAR and SPAR have not been calibrated.

Upcast

Filename:  u201120_xxxx.cnv  where xxxx is cast number

Upcast is supplied only as it provides a reference for unusual features seen in the 	downcast.

Standard seabird processing steps were used.

No spikes have been removed.

Pressure, primary and secondary temperature,  and primary and secondary conductivity have been calibrated.  Calibration coefficients and correction values applicable to oxygen downcast data were applied however the upcast’s strong hysteresis was not addressed making it unuseable.  Transmissivity drift was corrected.

Derived variables, salinity, potential temperature, sigma-theta and sound velocity, 	were recalculated.

Transmission, fluorescence, CDOM fluorescence, altimetry, ISUS, PAR and SPAR have not been calibrated.