EA Table 1.  List of m/z values used for internal calibration of (A) positive ion mode data and (B) negative ion mode data.  Exact mass refers to the mass calculated from the elemental formula, and charged mass is the exact mass value corrected for positive mode (by adding a Na atom and subtracting an electron) or negative mode (by subtracting a H atom and adding an electron).  For the positive ion mode data, we utilized Na adducts.  These compounds were chosen because of their frequent occurrence among the different samples analyzed in each mode, and their low error of observed m/z values (e.g. the error in mass accuracy ranged from 0.5 to 1.4 for the positive mode calibrants, and 0.3 to 0.8 ppm for the negative ion mode calibrants).  In positive mode, calibrants were present in at least six of the seven samples, and in negative mode, calibrants were present in at least half the samples. On occasion, calibrants were added for specific spectra when the original list of calibrants was insufficient to calibrate the desired mass range. In positive mode the internal calibrants span the full range of observed m/z values; whereas, in negative mode it was not possible to find calibrants above ~ 600 m/z that fit our criteria.  However, it is unlikely that the mass error of peaks outside our calibrated range fall outside the 1 ppm error set by the external calibrants because all of the negative mode samples were run within one week.

- Positive Mode Calibrants (Na Adducts)

|  | Elemental Formula | Exact Mass | Charged Mass |
| --- | --- | --- | --- |
| 1 | C8H18O5 | 194.115423 | 217.104642 |
| 2 | C10H22O6 | 238.141638 | 261.130856 |
| 3 | C12H26O7 | 282.167853 | 305.157071 |
| 4 | C17H36O6 | 336.251188 | 359.240407 |
| 5 | C24H38O4 | 390.277009 | 413.266228 |
| 6 | C18H38O10 | 414.246497 | 437.235715 |
| 7 | C20H42O11 | 458.272712 | 481.261930 |
| 8 | C22H46O12 | 502.298926 | 525.288145 |
| 9 | C24H50O13 | 546.325141 | 569.314360 |
| 10 | C26H54O14 | 590.351356 | 613.340574 |
| 11 | C28H58O15 | 634.377571 | 657.366789 |
| 12 | C30H62O16 | 678.403785 | 701.393004 |
| 13 | C37H68O12 | 704.471077 | 727.460296 |
| 14 | C35H62O16 | 738.403785 | 761.393004 |
| 15 | C42H86O15 | 830.596672 | 853.585890 |
| 16 | C45H92O16 | 888.638536 | 911.627755 |

- Negative Mode Calibrants

|  | Elemental Formula | Exact Mass | Charged Mass |
| --- | --- | --- | --- |
| 1 | C10H16O6 | 232.094688 | 231.087411 |
| 2 | C10H21O5N3 | 263.148120 | 262.140844 |
| 3 | C13H20O6 | 272.125988 | 271.118711 |
| 4 | C13H10O9 | 310.032481 | 309.025205 |
| 5 | C16H24O8 | 344.147117 | 343.139841 |
| 6 | C24H18O3N2 | 382.131742 | 381.124466 |
| 7 | C19H24O9 | 396.142032 | 395.134755 |
| 8 | C21H26O10 | 438.152597 | 437.145320 |
| 9 | C25H32O8 | 460.209718 | 459.202441 |
| 10 | C21H24O14 | 500.116605 | 499.109329 |
| 11 | C27H26O12 | 542.142426 | 541.135149 |
| 12 | C28H24O15 | 600.111520 | 599.104243 |
| 13 | C26H52O15 | 604.330620 | 603.323344 |