NTAS 8 Burn-in Notes

DATE:                      ACTIVITY:

10 DEC 07   Burn in begins.  Both primary loggers (L-12 & L-16) powered up with

HRH’s, LWR’s, and SST’s.  L-12 has SWR.  Both clocks set to UTC and

FLASH cards erased.  Buoy inside. PTT’s installed but not transmitting.

11 DEC 07   Both loggers powered down and the firmware updated to V3.21, then

powered back up.

12 DEC 07   L-05 powered up as the spare system, FLASH erased and clock set to

UTC.  HRH, LWR and SST on, and PTT off.  System is in high bay.

14 DEC 07   SWR501 plugged in to L-16 @ 18:11 UTC. SWR211 plugged in to the

spare (L-05).

18 DEC 07   BPR plugged in to L12 @ 15:50 UTC.

19 DEC 07   BPR216 plugged in to L-16 @ 13:54 and BPR217 plugged in to L-05 @

13:55 UTC.

31 DEC 07    All SST’s placed in a bucket @ 18:17 UTC.

1 JAN 08       L-12

Stop @ 13:19:30               File: N8L12_01.DAT = Records 1 to 16380

Records = 32638                       N8L12_02.DAT = Records 16350 to 32638

Restart @ 14:12:30

L-16

Stop @ 14:13:30               File: N8L16_01.DAT = Records 1 to 16380

Records = 32693                       N8L16_02.DAT = Records 16350 to 32693

Restart @ 15:08:30

L-05

Stop @ 18:03:30               File: N8L05_01.DAT = Records 1 to 16380

Records = 30182                       N8L05_02.DAT = Records 16350 to 30182

Restart @ 19:24:30

3 JAN 08     Both primary HRH’s removed for calibration checks @ 16:18 UTC.

9 JAN 08     PRC’s plugged in to all 3 systems and some cabling changed due to short

cables, and all done by 16:15 UTC.

14 JAN 08   L-12 ARGOS transmitter turned on @ 14:20, L-16 transmitter turned on @

14:25, and L-05 transmitter turned on @ 15:0o UTC. All 9 ID received

locally.

15 JAN 08   Unplugged L-05 PTT for re-programming @ 16:10 UTC.

16 JAN 08   HRH211 plugged in to L-12 @ 18:29 UTC.  HRH221 plugged in to L-16 @

18:34 UTC. Both HRH’s have new shields.