CalNex cruise

Requirements for Science Vans

Portable Science Vans 02 Level:

  - PMEL 1 (AeroChem Van)

wt	15,000 lbs

size	8' x 20'

power	70 amp 480 v three phase

location	outboard port side 02 level, sampling line connected to PMEL 2

Needs phone connection.

  - PMEL 2 (AeroPhys Van)

wt	17,000 lbs

size	8' x 20'

power	70 amp 480 v three phase

location	inboard port side 02 level, sampling mast mounted on roof

Needs phone connection, dry compressed air (120 psi, 4 CFM).

  - PMEL 3 (Guest Van)

wt	15,000 lbs

size	8' x 20'

power	Power from PMEL 1

location	port side 02 level aft of PMEL 1 and 2,

sampling line connected to PMEL 2

  - ESRL 1 (MABEL)

wt	15,000 lbs

size	8' x 20’

power	70 amp 480 v three phase

location	outboard starboard side, 02 level, inlets and grated decking mounted on roof

Needs phone connection

  - ESRL 2 (VOCs)

wt	15,000 lbs

size	8' x 20’

power	From ESRL 1 van

location	inboard starboard side, 02 level, inlets mounted on roof

Needs phone connection

  - ESRL HRDL

Wt		17,000

Size		8’ x 20’

Power		70 amp 480 v three phase

Location	aft 02 Level

Needs phone connection, RS422 serial line from the ship’s Seapath INU

Portable Science Vans 01 Level:

  - PMEL Alvan

wt	12,000 lbs

size	8' x 20'

power	30 amps 480 v  three phase

location	outboard, port side, 01 level

Needs freshwater line, Ethernet connection and phone.

  - PMEL spare parts/storage van

wt	12,000 lbs

size	8' x 20'

power	none

location	inboard port side, 01 level

Portable Science Vans Fantail:

  - ESRL 3 (pump van)

wt	7,200 lbs

size	8' x 8’

power	none

location	fantail

  - PMEL SW van

Wt 	12,000

Size	8’ x 16’

Power	70 amp 480 v three phase

Location	fantail

Needs freshwater & seawater lines, dry compressed air (120 psi, 4 CFM), phone connection

  - PSD Cloud radar van

wt	17,000 lbs

size	8’ x 20’

power	70 amp 480 v three phase

location	fantail

Needs phone connection

  - CSD Stg,  spare parts/storage van

wt	12,000 lbs

size	8' x 20'

power	none

location	Fantail

  - Univ VA

wt	15,000 lbs

size	8' x 20'

power	30 amp 240 V

location	Fantail

Needs seawater and freshwater lines, dry compressed air.