DRAFT Cruise Plan AR049

Date Submitted:	August 29, 2019

Platform:	RV Armstrong

Project Title:	ADEON: Atlantic Deepwater Ecosystem Observatory Network/  ROV Jason Engineering Operations

Project Dates:	November 30, 2020 to December 23, 2020

Prepared by:	________________________	Dated: __________________

Dr. Jennifer Miksis-Olds

ADEON Lead PI – Chief Scientist

University of New Hampshire

Approved by:	________________________	Dated: __________________

Matt Heinz

ROV Jason -  Chief Scientist

WHOI

Approved by:	________________________	Dated: __________________

WHOI Ship Ops

Title

Affiliation (Program or Lab)

Approved by:	________________________	Dated: __________________

Captain

Commanding Officer

I. 	Overview

A.	AR049 ADEON Cruise #5, November 30, 2020 to 23 December, 2020

This expedition is the last of five cruises for this project focused on recovery of ADEON landers at 6 sites, redeployment of new bottom landers at three sites along the outer continental shelf (OCS) of the United States east coast.  In addition to the lander work, station sampling will occur at each location consisting of CTD profiles, net trawls for biological specimens, and a fine-scale, lawnmower pattern active acoustic survey using the hull-mounted echo sounder on the RV Armstrong.  ROV Jason is joining the cruise to aid in recovering a lost lander at the ADEON JAX site that was not recovered during the AR040 cruise is November 2019. ROV Jason will be performing 2 engineering dives within the cruise schedule following its Fall 2020 overhaul.  This project is a collaboration among four federal agencies: National Science Foundation, Bureau of Ocean Energy Management (BOEM), NOAA, and the Office of Naval Research (ONR).  Data gathered during this mission in support of the ADEON project will help inform multiple management issues concerning this region. Details regarding daily operations and objectives can be found below.

B.	Days at Sea (DAS)

Cruise is scheduled (4-21 Dec) for 16 days of at sea operations.

C.	Operating Area (see Figure 1)

The cruise will focus on activities at seven station locations along the eastern US continental slope, 37.5° N, -80°W to 29°N, -74.5°W (See Figure 1, Table 1).

D.	Summary of Objectives

The goal of this expedition is to recover bottom landers at 6 locations, deploy bottom landers at 3 sites, collect additional active acoustic survey, CTD profiles, net trawl/tow sampling at each site, recover a disabled lander at JAX with ROV Jason, and perform 2 ROV Jason engineering dives. Specific objectives include:

- Recover 6 landers deployed during AR040 in November 2019.  (see Appendix 1)

- Conduct CTD casts to collect water column hydrography and water samples (down to 1000 m).

- Conduct net sampling for biological organisms using a mid-water trawl and vertical ring nets. (see Appendix 2)

- Collect echo sounder data using the EK80 system aboard the RV Armstrong during normal operations and during fine-scale (10 km x 10 km) lawnmower surveys at each study site.

- Two ROV Jason engineering dives

- ROV Jason operation to recover ADEON lander at JAX location

- Deploy 3 bottom landers at HAT, WIL, and SAV

E. 	Participating Institutions

1. University of New Hampshire

2. WHOI

3. Stony Brook University

4. JASCO Applied Sciences

F.	Personnel/Science Party: name, title, gender, affiliation, and nationality

|  | Name (Last, First) | Title | Date Aboard | Date Disem. | Gen. | Affi. | Nat. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Miksis-Olds, Jennifer | Chief Sci. - ADEON Lead PI AZFP/MMO | 12/2 |  | F | UNH | USA |
| 2 | Heintz, Matt | Chief Sci. – ROV Jason Ops | 11/30 |  | M | WHOI | USA |
| 3 | Warren, Joseph | ADEON Night Shift Lead Acoustics/Nets | 12/2 |  | M | SBU | USA |
| 4 | Blair, Hannah | Acoustics/Nets | 12/2 |  | F | SBU | USA |
| 5 | Lucca, Brandyn | Acoustics/Nets | 12/2 |  | M | SBU | USA |
| 6 | Lyons, Anthony | Landers/MMO | 12/2 |  | M | UNH | USA |
| 7 | Lawrence, Carmen | Landers | 12/2 |  | F | JASCO | USA |
| 8 | Milne, Grant | Landers/MMO | 12/2/ |  | M | UNH | USA |
| 9 | TBD | ROV Jason | 11/30 |  |  | WHOI | USA |
| 10 | TBD | ROV Jason | 11/30 |  |  | WHOI | USA |
| 11 | TBD | ROV Jason | 11/30 |  |  | WHOI | USA |
| 12 | TBD | ROV Jason | 11/30 |  |  | WHOI | USA |
| 13 | TBD | ROV Jason | 11/30 |  |  | WHOI | USA |
| 14 | TBD | ROV Jason | 11/30 |  |  | WHOI | USA |
| 15 | TBD | ROV Jason | 11/30 |  |  | WHOI | USA |

G.	Administrative

1.	Points of Contacts:

j.miksisolds@unh.edu			{Chief Scientist – ADEON Lead PI}

mheintz@whoi.edu			{Chief Scientist – ROV Jason}

joe.warren@stonybrook.edu		{ADEON Night Shift Lead}

2.	Diplomatic Clearances

None Required.

II.	Operations

The Chief Scientists are responsible for ensuring the scientific staff are trained in planned operations and are knowledgeable of project objectives and priorities.  The Commanding Officer is responsible for ensuring all operations conform to the ship’s accepted practices and procedures.

Key Changes and Points of Emphasis:

- Science watches should be on a regular schedule, but may be adjusted depending on tasks.

- Mission will Depart from and Return to Woods Hole, MA.

- Mobilization begins on 30 November 2020.

- CTD rosette must be on board, calibrated, with new bottles mounted; sensors rated to at least 1000 m.

- Would like to use the Armstrong’s USBL (I believe) transponder on our IKMT trawls (attached to wire just above our net) for real time net depth monitoring.

- EK80 will require current calibration (if available)

- Crane (probably ship’s) needed to load bottom lander, weight plate sets, and other items onto ship.

- Forklift needs to be on the dock and available for MOB/DEMOB

- DEMOB will start on 21 December upon vessel return.

- Project Itinerary:

This expedition will travel to seven site locations for deployment of bottom landers and water column (CTD, net, fine-scale acoustic survey) sampling.

Nov 30-3 Dec: 	MOB gear, personnel in Woods Hole, MA

For estimated time line of cruise operations, see Appendix 3.

21-23 Dec: 	DEMOB gear, personnel in Woods Hole, MA (expect that DEMOB will be done in 2 days)

- Staging and Destaging:

Ship’s personnel will be needed (e.g. Crane operator) to help load/offload scientific equipment during MOB/DEMOB.

C.	Operations to be Conducted:

Operational Plans: The cruise consists of a series of station operations (e.g. lander deployment, ctd profile, etc) that will occur at each station.  As well as a few operations that will occur once (ROV Jason engineering dives, ROV Jason JAX recovery).  Detailed information about the non-standard different operations are contained in the Appendices to this cruise plan.

Scientific Computer System (SCS):    We request standard SCS data collection for the cruise.  This includes standard surface hydrographic measurements (temperature, salinity, fluorescence, etc).

EK80  Data Acquisition: The Simrad EK80 echo sounder, (18-, 38-, 70-, 120-, and 200-kHz with split-beam transducers) will acquire data continuously throughout the cruise, as long as its operation won’t interfere with other sensor data collection. The EK80 will be interfaced to the SCS to record bottom depth and vessel log values. The EK80 will be interfaced to the POSMV motion sensor. When operational, the EK80 will be synchronized with the Simrad ME70 multibeam (operating within the frequency band 70–120 kHz), and the ship’s ADCP. The EK80 is not synchronized with the other sounders and Doppler speed log on the vessel. To minimize acoustic and electrical interference, whenever possible we request deactivating other sounders on the vessel. The survey technicians will be responsible for EK80 and ME70 data acquisition and storage.

Further details on these additional operations are as follows:

1.	Physical oceanographic parameters will be monitored through CTD casts, and the ship’s flow-through thermosalinograph and fluorometer instruments. Both of these systems (flow-through and CTD rosette-based) contain fluorescence sensors which will be used to provide information on horizontal and vertical variations in primary productivity. These data will be used to help reference and integrate the satellite measurements of primary productivity that are also being integrated into the ADEON data repository.

2. 	CTD casts will be conducted when at each site.  The ship will supply SBE19+ and SBE911+, ECO Chlorophyll Fluorometer and Turbidity sensor (rated to at least 2000 m), and O2 sensor (rated to at least 2000 m). CTD rosette will also collect discrete water samples. The survey technician, with help from the science team, will be responsible for CTD operations. Some water samples may be filtered to ground-truth primary productivity estimates made from the fluorescence sensors on the CTD rosette and ship’s flow-through system.

4. 	Mid-water trawling will be conducted to sample deep-scattering layers identified in the EK80 data. The science team, with help from the survey tech and deck crew, will be responsible for trawling operations. (see Appendix 2)

D.	ROV Dive Plan

Transit to 3km site.

Cast to test winch and crane and relax and recharacterize cable

- 280 lb. dummy weight, boot cable ends

- Rotation sensor 10m wire out

- Rotation sensor & USBL 11m wire out

- Rotation sensor 100m wire out

- Rotation sensor 101m wire out

- Go to 2900m wire out, don’t touch bottom

- Depth on site is 2997m

- Produce characterization curve for use on future dives

- Functional test of LARS

  - Docking head and winch

  - Rapp crane control box

  - 2x AHC tests using Norsub and Kongsberg MRUs compare

Transit to Miksis-Olds work site

- Connect Jason and recover JAX lander

Deploy elevator with usbl

- Test USBL

  - Track Elevator

  - Determine if IMU is functional

  - Determine if Cassius is required

- Jason functional tests

- Manipulators functional test

- Test all cameras, confirm image quality

- Test and Adjust lighting as needed

- Test new Telemetry system

- Test prototype thruster

- Test Rapp Winch

- Test NPC crane

- Blue View tests

- Multibeam tests

  - Collect data as per Jennifer doc

  - drive pattern to estimate the roll bias

- AFX power system testing

- Test borrowed strobes on Jason

- Test optical modems

- Test Tele-presence

  - Sealog operations from shore, latency etc

  - Transmit and process imagery on shore

- Pilot training for new team members

- Confirm condition of cable torque on first dive

- Test spare joy box

- Install new connectors on spare MC to test if it resolves connector failures, time permitting

E.	Applicable Restrictions

Bad weather conditions, high sea states, equipment failure, safety concerns, and unforeseen circumstances can preclude normal operations. The ship’s officers, chief scientist, and other personnel will assess and address any concerns or issues affecting normal operations.

III.	Equipment

- Equipment and Capabilities provided by the ship (itemized)

CTD rosette with sufficient weight to conduct CTD casts down to 2000m

CTD sensors required: transmissometer, DO, temperature, conductivity, 	fluorescence, depth

EK80

Multibeam SONAR

ADCP

Dynamic Positioning

Oxygen sensor rated to at least 2000m

ECO Chlorophyll Fluorometer and Turbidity sensor rated to at least 2000m

USBL transponder (for attachment to trawl wire above our mid-water trawl)

Working winches

Working cranes

Freezers—80 and walk-in freezer

Seawater available on deck

- Equipment and Capabilities provided by the scientists (itemized)

Net tow systems.

Bottom landers and weight plate sets. (see Appendix 1)

IV.	Hazardous Materials

A.	Policy and Compliance

The Chief Scientists will include with their project instructions and provide to the CO of the respective ship 30 days before departure:

    - List of chemicals by name with anticipated quantity

    - List of spill response materials, including neutralizing agents, buffers, and absorbents

    - Chemical safety and spill response procedures, such as excerpts of the program’s Chemical Hygiene Plan or SOPs relevant for shipboard laboratories

    - For bulk quantities of chemicals in excess of 50 gallons total or in containers larger than 10 gallons each, notify ship’s Operations Officer regarding quantity, packaging and chemical to verify safe stowage is available as soon as chemical quantities are known.

B.  Inventory

| Common Name of Material | Qty | Notes | Trained Individual | Spill control |
| --- | --- | --- | --- | --- |
| Formalin | 10 liters | Stored in ship chem. locker | J. Warren | F |
| 95% Ethyl Alcohol (ethanol) | 2 liter | Alkalinity, stored in ship chem. locker | J. Warren | E |
| Borate buffer (Borax) | 1 kg | Stored in ship chem. locker | J. Warren | NA - powder |

C.  Chemical safety and spill response procedures

F: Formalin/Formaldehyde

- Ventilate area of leak or spill. Remove all sources of ignition.

- Wear appropriate personal protective equipment.

- Isolate hazard area. Keep unnecessary and unprotected personnel from entering. Contain and recover liquid when possible.

- Use non-sparking tools and equipment. Collect liquid in an appropriate container or absorb with an inert material (e. g., vermiculite, dry sand, earth), and place in a chemical waste container.

- Do not use combustible materials, such as saw dust.

E: Ethanol

- Ventilate area of leak or spill. Remove all sources of ignition.

- Wear appropriate personal protective equipment.

- Stop the flow of material. Dike the spilled material.

- Carefully cover the area with spill absorbent material.

- Sweep up the residue using spark-proof tools and place the residue into a labeled, plastic, waste container (plastic pail with lid or double heavy duty plastic bags). Store for disposal as hazardous waste.

- Mop the affected area using detergent and water.

D.	Radioactive Materials

No Radioactive Isotopes are planned for this project.

E.	Inventory (itemized) of Radioactive Materials

Not applicable

V.	Additional Projects

A.	Supplementary (“Piggyback”) Projects

No Supplementary Projects are planned.

VIII.	Appendices

X

Figure 1. Map of ADEON site locations and ROV Jason engineering dives (star) to be visited during the RV Armstrong AR049 cruise.

Table 1: Lander deployment locations, depth and estimated bottom locations from the AR040 cruise.  Recovery station locations for AR049 cruise should be based on the “Estimated Location” column in this table.  Deployment station locations for HAT, WIL, and SAV should be on the “Deployment Location”.

| Site |  | Deployment Location |  |  | Drop Date |  | Time (UTC) |  | Estimated Location |  |  |  | Depth (m) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Latitude |  |  | Longitude |  |  |  |  |  | Latitude |  | Longitude |  |  |
| BLE |  |  |  |  | 2019-10-28 |  | 14:58 |  | 29° 15.0539’ N |  | 078° 21.0547’ W |  | 900 |  |
| CHB |  |  |  |  | 2019-10-31 |  | 09:29 |  | 32° 04.2062’ N |  | 078° 22.4236’ W |  | 415 |  |
| SAV |  | 32° 02.53’N | 077° 20.87’W |  | 2019-10-25 |  | 19:58 |  | 32° 02.5275’ N |  | 077° 20.7537’ W |  | 814 |  |
| WIL |  | 33° 35.11’N | 076° 27.033’W |  | 2019-10-24 |  | 16:37 |  | 33° 35.2176’ N |  | 076° 26.9743’ W |  | 464 |  |
| HAT |  | 35° 11.97’N | 075° 01.23’W |  | 2019-10-23 |  | 17:07 |  | 35° 11.9927’ N |  | 075° 01.1821’ W |  | 294 |  |
| JAX |  |  |  |  | 2019-01-12 |  | 17:42 |  | 30° 29.7180 |  | 080° 00.1322 |  | 324 |  |
| JAX ROV Recovery |  |  |  |  | 2018-11-07 |  | 17:52 |  | 30 o 29.6168 N |  | 80 o 00.1598 W |  | 317 |  |
| ROV Jason Eng. Dives |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Appendix 1. Bottom-Lander Deployment and Recovery

The mooring has no cable (except for the winch used to lower it to the water surface), it is a freefall tetrahedral frame. It contains one component, with the anchor built into the structure. There will be 8 altogether, and I've attached a drawing, which contains the dimensions. Each mooring weighs ~ 550 kg and are subsurface with no surface impression.

Figure A1.1. Schematic of bottom lander tetrahedral free-fall frame.

We will be supplying our own deck boxes to communicate with the acoustic releases.

The deployment procedure is as follows:

- 1. Vessel will approach the deployment position, down wind and current. Lander will be hung abaft the A-frame (complete mooring) via centerline winch. A-Frame will be used for over boarding the Lander, ~550kg with an on-load quick release (Sea Catch TR7 model). A slip-through tag line will be passed around the starboard side of the A-frame to control motion of lander during lifting.

- 2. Lander will be lowered below the water-line until the on-load release is just above the waterline. Vessel to drift into deployment position.

- 3. Ship is to maintain minimum speed to maintain position.

- 4. Once the vessel is in position, the tag line may be removed by the JASCO Lead. Once the tag line is retrieved, the order to release the on-load release and deploy the Lander via A-Frame will be given once all personnel are in position. All non-essential personal will be off the aft deck.

- 5. The order to ‘release’ will be given by the JASCO Lead.

- 6. Once the mooring is on bottom, JASCO to annotate logs, ship to mark plot.

- 7. Once the Lander is bottomed, a range check with the PortLF release will be done and position in logs will be annotated. Release is then disabled.

- Retrieval Procedure:

  - Ship is to approach to an upwind position ~300m from the AEDON Lander waypoint and maintain position.

  - Transducer is lowered, ~15m below the waterline, ship de-clutched. Enable code is sent and JASCO technician establishes communication and range with PORT LF acoustic release. Once communication and position are established, release code is sent.

- With FRC:

  - Once the ADEON buoy is on the surface the FRC crew deploys and both vessel and FRC begins steaming towards lander.

  - Crane operator lowers hook to the water line and FRC crew attaches crane hook to lift point of the lander. FRC crew clears area and crane operator begins lifting lander aboard.

- Without FRC:

  - Once the ADEON buoy is on the surface vessel steams toward location. Crew prepares Kong boat hook attached to gaff with loop in line to attach to crane hook.

  - Once vessel reaches lander, crew member uses gaff to snap Kong boat hook onto the lift point of the lander and detaches Kong hook from gaff. Once gaff is free, the loop in the lift line on end of Kong hook is attached to the crane hook and the lander is lifted aboard and secured.

Sea Catch TR7 On-Load Quick Release (left) Kong Boat Hook (right)

Appendix 2.	Net trawl operations

Mid-water Trawls: An Isaacs-Kidd Midwater Trawl (IKMT) and smaller ring nets will be used to collect midwater organisms for community analysis and to ground truth the acoustic data (Fig. A2.1).

The IKMT net is approximately 2 m wide by 3 m high (when fully extended) by 7 m in length.  It consists of a horizontal spreader bar at the top and a weighted, depressor plate or v-fin at the bottom which are connected by a wire bridle that is collapsible.  The net can be deployed from the stern or side of the vessel and has a single cod-end which is deployed/recovered by hand. Securing the weighted depressor v-fin  is the primary launch and recovery consideration.

The IKMT is connected to a standard 9/16” trawl wire.

Multiple tows to specific depth-layers may conducted and would generally occur  to depths ranging from 100 to 2000 m.  In most cases the wire-out/net depth scope is ~1.7:1 to 2:1, so the maximum wire out needed is estimated to be 3000 m.  Wire out/in speeds range from 10 to 30 meters per minute. Tows are conducted at vessel speeds of 2-4 kts and will take approximately 60-120 minutes depending on layer depth.

On our previous Armstrong cruise, the Armstrong’s USBL (I believe) transponder was attached to the trawl wire just above our net which allowed us to have real-time depth-monitoring while doing the tow which was INCREDIBLY useful. We’d like to use that again if possible.

Ring Nets: Vertical net casts will be made using a bongo-type net.  These will be made with the ship stationary.  Net mouth diameters will be ~60 cm or smaller and 5-7 m in length and can be deployed by hand via line or preferably by shackle attachment to a winch with a large weight (30+ lbs) attached below the net.  Nets are lowered to a depth and then hauled back (typical downcast wire out speeds are 30 m/min and haul back speeds are 10-15 m /min).  These can be done preferably from the side of the ship rather than the stern. If the ship has a clump weight (15 kg+), we would use that to keep the cable vertical during the net haul.

Figure A2.1 An IKMT net (left) and bongo-net ring nets (right) being deployed off the stern (left) and starboard side (right) during EN 615.

Appendix 3.	Timeline of cruise operations.  ADEON/ROV Jason Preliminary Cruise Plan (as of 2020.08.25) for Dec 2020 (AR049) RV ARMSTRONG