Executive Summary: Modeling and Analyzing the Propagation from Environmental through Sonar Performance Prediction

W. A. Kuperman, W. S. Hodgkiss, B. D. Cornuelle,, K. Kim and A. Thode

Scripps Institution of Oceanography

La Jolla, CA  92093-0701

phone: (858) 534-7990 / (858) 534-1798  fax: (858) 822-0665

email: wak@mpl.ucsd.edu / wsh@mpl.ucsd.edu

Jeffrey Krolik and Loren W. Nolte

Duke University

Henry Cox and Kevin D. Heaney

ORINCON Corporation

Michael B. Porter, Paul Hursky, and M. Siderius

Science Applications International Corporation

Award Number: N00014 –01-1-0768

LONG-TERM  GOALS

Central to the long term goals of this joint project was to understand the physics of the propagation of uncertainty through the interfaces between oceanography, acoustics, array processing and performance prediction.

### OBJECTIVES

The objective of this research program wasto both develop a methodology to predict uncertainty in the whole performance prediction process and to understand the uncertainty physics of the individual components of the process. The latter provides the potential to develop methods to reduce uncertainty.

### WORK COMPLETED

The results are reported in the  year end reports of the individual  team components of SIO/MPL, Duke, SAIC and Orincon. Below is a tabulation of the final status of the results. More detail is provided in the individual reports.

- Development of acoustic/pressure adjoint methodology.

- Development of Rapid 3D modeling/sensitivity Monte Carlo capability.

- Extension of adjoint theory beyond pressure to other observables.

- ROMS configured for 600 m nested grid in SwellEx region,

- Including tidal forcing; Realizations generated from ROMS.

- Pre-assimilation studies exploring ensemble generation and nonlinearity and information content of intensity data.

- Related striation inversion with TL uncertainty; combined geo and ocean acoustics uncertainty --> detection range uncertainty.

- Fast, analytical methods for characterizing the performance of both optimum Bayesian and adaptive CFAR detectors have been derived.

- Analytical methods derived that approximate the ROC for optimum Bayesian detection for an uncertain ocean environment model in terms of the signal covariance matrix rank, SNR, and a signal-to-interference coefficient.

- Recent for the multiple source SWellEx-96 S59 event using the AODS North horizontal array yields agreement between theory and real data detection performance when wavefront uncertainty and limited snapshot support are accounted for in the procedure.

### IMPACT / APPLICATIONS

This expected impact of this project is to provide a methodology to provide a reliability measure to the operator of at-sea performance prediction models. Some of the research results are now being applied to the study and performance prediction of potential passive acoustic distributed systems. Rapid geophysical inversion methods was also developed. Further, the impact of array processing was addressed to the point where some of the material has been requested to be presented to the signal processing working group considering insertion of algorithms into existing systems.

### PUBLICATIONS

The combined publication list of the total team (not including abstracts) is given below:

- P. Hursky, M. B. Porter, B. D. Cornuelle, W. S. Hodgkiss, W. A. Kuperman, “Adjoint-Assisted Inversions for Shallow Water Environmental Parameters,” in Impact of Littoral Environmental Variability on Acoustic Predictions and

- Sonar Performance Eds: N. Pace and F. Jensen, Boston: Kluwer Academic (2002).

- P. Hursky, M. B. Porter, B. D. Cornuelle, W. S. Hodgkiss, W. A. Kuperman, “Adjoint modeling for acoustic inversion,” J. Acoust. Soc. Am. 115,607-619 (2004).

- A. Thode and K. Kim, "An adjoint normal-mode approach for computing

- derivatives of pressure with respect to environmental parameters and frequency," J. Acoust. Soc. Am. (2003).

- A. Thode, "A normal-mode formula for the derivative of a waveguide pressure field with respect to three-dimensional sound speed perturbations,” J. Acoust. Soc. Am. (2004).

- Kevin Heaney, Henry Cox, “ A Tactical Approach to Environmental Uncertainty and Sensitivity” IEEE - JOE Henry Cox, Submitted, May 2004

- P Gerstoft, CF Huang ,and WS Hodgkiss, “Posterior estimation of transmission loss from ocean acoustic data ,” submitted IEEE  J. OceanicEngineering,2004.

- P Gerstoft, CF Huang and W.S. Hodgkiss: “Aposterior transmisson loss from ocean acoustic data,” 6th Euro. Conf. Und Ac. Delft 2004.

- Liewei Sha and Loren W. Nolte, “Detection performance prediction in uncertain ocean environments:  Bayesian viewpoints”, Journal of the Acoustical Society of America, revision submitted April 2004.

- Liewei Sha and Loren W. Nolte, “Bayesian sonar detection performance prediction in the presence of interferences in uncertain environments”, Journal of the Acoustical Society of America, submitted December 2003.

- Liewei Sha and Loren W. Nolte, “Bayesian sonar detection performance prediction with SWellEX96 vertical array data”, special issue on Capturing environmental uncertainty in sonar performance, IEEE Transactions on Oceanic Engineering, submitted February, 2004.

- Liewei Sha and Loren W. Nolte, “Localization performance prediction in uncertain ocean environments; Bayesian viewpoint and analytical approximations”, special issue on Capturing environmental uncertainty in sonar performance, IEEE Transactions on Oceanic Engineering, submitted February, 2004.

- Loren W. Nolte, Stacy L. Tantum, and Liewei Sha, (invited paper) “Bayesian Matched Field Tracking, Depth Classification, and Environmental Inversion in Ocean Acoustics”,  Sixth International Conference on Theoretical & Computational Acoustics, Honolulu, Hawaii,  April 11-15, 2003.

- J. L. Krolik, P. Book, S. Kraut, “In situ detection performane assessment of a sonar array when the signal wavefront and noise covariance are uncertain”, Proc. of Sensor Array and Multichannel Signal Processing Workshop, Barcelona, July 2004 (in press).

- J.L. Krolik, P. Book, and S. Kraut, “In Situ Adaptive CFAR Performance Assessment for Passive Sonar” in preparation for submission to the IEEE Journal of Oceanic Engineering, 2004.